IP addressing & Subnetting

Task 1 :- List the IP address classes (A, B, C, D, E) and give one real-world example of a device or service that might use each class. Write your answers in a table format.

ANS :-

| Class | Range / Primary Use | Who or What Uses It ( Example) |
| --- | --- | --- |
| Class A | Huge networks (1.0.0.0 – 126.255.255.255) | Tech giants & ISPs (e.g., Apple owning the entire 17.x.x.x IP block for its global infrastructure) |
| Class B | Medium-to-large networks (128.0.0.0 – 191.255.255.255) | Universities & large companies (e.g., MIT or large corporate office networks) |
| Class C | Small networks (192.0.0.0 – 223.255.255.255) | Home Wi-Fi routers (e.g., your home router assigning 192.168.1.5 to your smartphone) |
| Class D | Multicasting / Group delivery (224.0.0.0 – 239.255.255.255) | Live IPTV / Video streaming (e.g., broadcasting live cable channels to multiple set-top boxes at once) |
| Class E | Reserved (240.0.0.0 – 255.255.255.254) | Network researchers & testing labs (e.g., R&D engineers developing future networking protocols) |

Task 2 :-  Given the IP address 192.168.10.45 with subnet mask 255.255.255.0, calculate the network address, broadcast address, and the range of valid host addresses.

ANS :-

Network Address : 192.168.10.0

Broadcast Address : 192.168.10.255

Valid Host Address Range : 192.168.10.1 – 192.168.10.254

Step-by-Step Breakdown :-

- Network Address : Performing a bitwise AND operation between the IP address (192.168.10.45) and the subnet mask (255.255.255.0) leaves the network portion intact while setting all host bits to 0.

  - Result: 192.168.10.0

- Broadcast Address : Setting all host bits (the last octet) to 1 (or 255).

  - Result: 192.168.10.255

- Valid Host Range : Any address strictly between the network address and the broadcast address.

  - First usable host: 192.168.10.1

  - Last usable host: 192.168.10.254

Task 3 :- Create a subnetting plan for a small food delivery startup like Zomato that needs 4 separate subnets: one for Delivery Partners, one for Restaurants, one for Customers, and one for Admins. Assign a unique subnet (with network address and subnet mask) for each group using the 10.0.0.0/24 network.

ANS :-

To split the base network 10.0.0.0/24 into 4 equal subnets, we borrow 2 host bits (22=4 subnets). This changes the subnet mask prefix from /24 to /26 (255.255.255.192).

Each subnet provides 62 usable IP addresses for connected devices and services.

*Subnetting Allocation Plan :-

| Department / Group | Network Address | Subnet Mask | CIDR Notation | Usable Host IP Range | Broadcast Address |
| --- | --- | --- | --- | --- | --- |
| Delivery Partners | 10.0.0.0 | 255.255.255.192 | 10.0.0.0/26 | 10.0.0.1 – 10.0.0.62 | 10.0.0.63 |
| Restaurants | 10.0.0.64 | 255.255.255.192 | 10.0.0.64/26 | 10.0.0.65 – 10.0.0.126 | 10.0.0.127 |
| Customers | 10.0.0.128 | 255.255.255.192 | 10.0.0.128/26 | 10.0.0.129 – 10.0.0.190 | 10.0.0.191 |
| Admins | 10.0.0.192 | 255.255.255.192 | 10.0.0.192/26 | 10.0.0.193 – 10.0.0.254 | 10.0.0.255 |

How the Calculation Works :-

- Original Block: 10.0.0.0/24 (256 total IP addresses: 10.0.0.0 to 10.0.0.255).

- Subnet Increment Size: With a /26 prefix, each subnet spans 64 total addresses (256÷4=64).

- Network Boundary Step: Network addresses start at multiples of 64 (0, 64, 128, 192).

- Usable IPs: The first IP in each range is reserved for the Network Address, and the last IP is reserved for the Broadcast Address. The remaining 62 IPs are assignable to devices or servers.

Task 4 :- You are given the following scenario: A college WiFi network uses the IP range 172.16.0.0/16. The admin wants to create at least 20 subnets for different buildings. Calculate the new subnet mask and the number of hosts available per subnet.

ANS :-

To get at least 20 subnets, the admin needs to borrow 5 bits from the host portion (25=32 possible subnets).

- New Subnet Mask: 255.255.248.0 (or /21)

- Usable Hosts per Subnet: 2,046

Quick Breakdown :-

- Borrowing Bits: Moving from /16 to /21 adds 5 bits to the network part. 25=32subnets, which covers the 20 required.

- Host Calculation: 11bits remain for hosts (32-21=11). 211-2=2,046usable IP addresses per building.

Task 5 :- A friend sends you a message asking why their device with IP 192.168.1.300 is not connecting to the network. Write a short explanation identifying the error and what a valid IP address in that subnet should look like.

ANS :-

The number 300 is impossible in an IP address.

In IPv4, each of the four numbers can only go up to 255 (from 0 to 255).

What it should look like :-

A valid IP in that subnet must replace the last number with anything from 1 to 254 .

for example: 192.168.1.30  or 192.168.1.100